[gds_Brevet_Blocking_Strike]

Grande Sonnerie	Patent

Blocking the strike automatically

05405513.2 / EP05405513

The chimes are automatically blocked to reserve the last 24 hours of the mainspring’s energy to the movement.

A cam (61), working off the power-reserve differential, switches the pivoting lever (18) when the power reserve falls to 24 hours. The blocking device (80) keeps the clutch-pinion (47) meshed with the driving pinions of the rack (44, 45), thus preventing the racks from falling to start the chime.

When the power-reserve exceeds 24 hours, the cam releases the blocking mechanism (80), returning the pivoting clutch (46, 47) to the control of the rocking arm (48).