[gds_Brevet_Blocking_Strike_Winding]

Grande Sonnerie	Patent

# Blocking the strike and the winding stem

05405509.0 / EP05405509

This safety system prevents the winding-stem from being pulled out when the strike is operating, and it blocks the strike when the stem is pulled out.

A locking-cam (66) can be pivoted in two positions: one to block the manual strike-release button (63); the other to lock the winding-stem (9) by acting on (66d). Pulling the winding stem out pushes the lever (65) to the left, pivoting the locking-cam (66) so that it blocks the manual strike-release (62b).

The locking cam is also switched by a pivoting sprung arm (43) connected to the hours-rack. As soon as the rack moves to begin the strike, the arm (43) pivots the cam (66) so that it engages the stem-blocking device (66d) and prevents the stem from being pulled out to set the time