[gds_Brevet_Chimings_Gongs]

Grande Sonnerie	Patent

# Chiming gongs

05405507.4 / EP05405507

This new design produces a louder, clearer sound from gongs that are just three-tenths of a millimetre thick. Mounting the gongs on the movement instead of around it increases the diameter of the movement.

The gongs for a chiming mechanism consist of a flat blade (1) one end of which (1a) can be fastened to the structure of the watch. The free part of the blade is shaped so that its natural frequency is audible when its edge is struck.