[gds_Brevet_Strike]

Grande Sonnerie	Patent

# Strike selection

05405508.2 / EP05405508

Controlled by a column wheel, the mechanism to select and indicate the strike mode performs its duties with the minimum of parts.

The rotating column-wheel (67) acts on three levers: the lever to select grand strike or small strike (73), the lever to select silence (72) and the sprung rocking-arm (69) that indicates the selected strike mode on the dial via a rack and pinion (69a, 71).

Pressing the strike-select button (F) makes rocking-arm (68) lift the sprung pawl (68a). The pawl pulls the 12-toothed wheel (67a) step by step to rotate the column-wheel through consecutive cycles of actions.