[gds_Brevet_Strike_Rack]

Grande Sonnerie	Patent

# The striking racks

05405512.4 / EP05405512

The construction of the movement, with the hours and minutes off-centre, allows the striking-racks to be mounted in the centre of the movement. This in turn makes possible outsize racks for greater precision and control.

The strike is governed by three coaxially mounted racks: for the hours (37), for the quarters (38) and for the minutes (39). Each has ratchet toothing (37a, 38a, 39a) to activate the hammers. When the strike is released, the racks drop on their respective cams. They are then immediately engaged by the strike-train and lifted to the end of their travel. As the racks lift, their outer teeth (37a, 38a, 39a) engage the gathering-pallets to strike the gongs. The distance each rack is lifted, thus number of teeth presented to activate the strike, depends on its starting point on its cam.