[gds_Brevet_Winding_Setting]

Grande Sonnerie	Patent

# Winding and setting system

05405506.6 / EP05405506

The compact winding and setting system does away with the long stem and sliding-pinion, and finds its place under the dial.

In this system, the crown-wheel (8) is permanently meshed with the transmission-wheel (7) mounted coaxially on a rocking arm (6). The winding-stem (9) slides through a square in the centre of the crown-wheel (8).

The transmission-wheel (7) drives intermediate-wheels for winding (5) and for setting the time (10), mounted at either end of the rocking arm (6).

Pulling out the winding-stem (9) makes the pullout piece (14) pivot the arm (6) to mesh the intermediate-wheels (10) with the minute-wheel (11) for setting the hands.

With the stem (9) in the winding position, the torque produced by the winding action on the transmission wheel (7), swings the rocking arm (6) to mesh intermediate wheel (5) with the winding ratchet.