[gds_Caracteristiques]

Specifications

MOVEMENT

Caliber 1505

Manually wound

Gold baseplate and bridges

40 jewels

MOVEMENT DIMENSIONS

Overall diameter: 	35.80mm

Casing diameter: 	35.00mm

Frame height: 	6.25mm

Overall height:	7.80mm

Height of stem axis: 	3.41mm

Stem-thread diameter: 	S1.20mm

BALANCE

Free-sprung

4 adjustment weights

Flat Anachron balance-spring

Fixed stud-holder

Pinned stud

Spring pinned to the collet

Frequency:	21,600 v/h (3Hz)

Inertia:  	11.00 mg/cm2

Angle of lift: 	52°

Amplitude:	Dial up, fully wound: > 340°

Dial up, 24 hours: > 300°

ESCAPEMENT

Linear escapement

15-tooth escape-wheel

VISUAL INDICATIONS

Hours and minutes, off-centre

Small seconds at 6 o’clock

Power reserve

Strike mode selection

Striking hammers

SOUND INDICATIONS

Grand strike, small strike on the quarters in passing

Strike silent

Minute-repeater on demand

AUTONOMY

120 hours without chime

Approx. 48 hours with grand strike

Approx. 24 hours after strike runs down

FINISH

Circular grained baseplates

Circular ribbing on the bridges

Polished and bevelled screws

Pegs with rounded, polished ends

Steelwork decorated by hand

3D engraving

CASE

Steel

Diameter:	42.00mm

Overall height:	12.25mm

CONTROLS

Two position winding and setting crown

Pushbutton to release the minute-repeater at 2 o’clock

Pushbutton to select the strike mode at 4 o’clock

NÚMBER OF PIECES

Movements without dial	408

Cased-up, on leather	441

Cased-up, steel bracelet	558